AI-Driven IoT Temperature Control System for Saffron Cultivation: A Comprehensive Review

[Author Name], [Author Name]

[Department Name], [Institution Name], [City, Country]

Email: {author1, author2}@institution.edu

Abstract — Saffron (*Crocus sativus* L.) is among the world's most economically valuable spice crops, with productivity critically dependent on precise microclimate temperature management. This paper presents a comprehensive review of a low-cost, AI-integrated Internet of Things (IoT) system designed for real-time environmental monitoring and automated climate control in saffron cultivation. The architecture comprises two ESP32 microcontrollers—one interfaced with a DHT22 temperature and humidity sensor, and one driving a relay-controlled cooling fan—coordinated through a Python Flask RESTful backend. A Long Short-Term Memory (LSTM) neural network, trained on a domain-specific saffron temperature dataset with temporal feature engineering (cyclic time encoding, multi-lag inputs), serves as the predictive core of the system. Fan actuation is governed by a hysteresis-based closed-loop control algorithm comparing real-time sensor readings against LSTM-predicted optimal setpoints. Empirical evaluation on a held-out test set of 2,896 samples demonstrates high predictive accuracy: Mean Absolute Error (MAE) of 0.3523°C, Root Mean Squared Error (RMSE) of 0.4960°C, and R² of 0.9920. The system further provides real-time monitoring through a web-based dashboard. This review covers the architecture, machine learning pipeline, control logic, experimental results, and future research directions for intelligent precision-agriculture systems.

Index Terms — Internet of Things, LSTM, Saffron Cultivation, Precision Agriculture, Temperature Control, ESP32, Deep Learning, Smart Greenhouse.

I. Introduction

The convergence of embedded computing, wireless networking, and deep learning is redefining precision agriculture. The Internet of Things (IoT) enables fine-grained, real-time sensing of environmental parameters, while machine learning provides the predictive capability to anticipate and pre-empt adverse conditions rather than merely react to them [1]. These combined capabilities are particularly relevant for high-value specialty crops with narrow environmental tolerances.

Saffron (Crocus sativus L.), derived from the stigmas of its flower, commands prices exceeding $3,000 USD per kilogram and represents a major agricultural export for regions including Iran, Kashmir (India), and Spain [2]. Its cultivation is exquisitely sensitive to ambient temperature: deviations during the critical flowering period (October–November) directly impair stigma development, pigment synthesis, and overall yield. Conventional greenhouse climate management relies on reactive, threshold-based thermostats that introduce inevitable lag between a temperature deviation and its correction.

A predictive approach—one that anticipates optimal temperature setpoints based on time-of-day and seasonal patterns—can reduce thermal stress episodes and improve crop consistency. This work presents and reviews such a system, integrating:

A distributed ESP32-based sensor-actuator network communicating over Wi-Fi.

A DHT22 sensor for high-accuracy temperature and humidity acquisition.

An LSTM neural network for time-series temperature prediction with temporal feature engineering.

A Flask RESTful API implementing closed-loop control with hysteresis.

A web dashboard for live monitoring and historical analysis.

The remainder of this paper is structured as follows: Section II surveys related work; Section III details the system architecture; Section IV describes the machine learning model; Section V presents the control logic; Section VI reports experimental results; Section VII offers discussion; Section VIII outlines future work; and Section IX concludes the paper.

II. Related Work

A. IoT in Precision Agriculture

IoT-based agricultural monitoring has been extensively explored. Gondchawar and Kawitkar [3] proposed an Arduino-based smart agriculture system integrating soil moisture, temperature, and humidity sensors with wireless communication. Patil and Kadu [4] demonstrated automated IoT irrigation control responsive to real-time soil conditions. Broader surveys by Tzounis et al. [5] identify key challenges including energy harvesting, connectivity reliability, and data management scale. The ESP32 platform, offering dual-core processing, integrated Wi-Fi/Bluetooth, and a rich peripheral set at sub-$5 cost, has become pervasive in agricultural IoT prototypes [6].

B. Deep Learning for Environmental Forecasting

Recurrent Neural Networks (RNNs), particularly LSTM networks introduced by Hochreiter and Schmidhuber [7], are the standard neural architecture for sequential time-series forecasting, owing to their gated cell mechanism that selectively retains long-range temporal dependencies. In agricultural contexts, LSTM models have been applied to soil temperature prediction [8], greenhouse microclimate forecasting [9], and evapotranspiration estimation [10]. The use of sine-cosine cyclic transformations for periodic temporal features (hour, month) is an established best practice to prevent artificial boundary discontinuities in the input representation [11].

C. Saffron Agronomy and Climate Requirements

The environmental requirements of Crocus sativus are well-characterised [2], [12]. Corm dormancy (June–August) tolerates temperatures of 25–35°C, while the critical sprouting (15–20°C) and flowering (5–15°C at night, 15–20°C daytime) stages demand precise thermal regulation. Molina et al. [12] demonstrated that temperature deviations of as little as ±5°C during flowering measurably reduce yield quantity and saffron quality. This narrow operational window motivates precision climate control.

III. System Architecture

The proposed system follows the canonical three-tier IoT architecture: a Perception Layer (sensors and actuators), a Transport Layer (Wi-Fi / HTTP REST), and an Application Layer (AI inference, control logic, web interface). Fig. 1 provides the complete system block diagram.

┌──────────────────────────────────────────────────────────────┐

│              APPLICATION LAYER                               │

│  ┌─────────────────────────────────────────────────────────┐ │

│  │           WEB DASHBOARD (HTML / JavaScript)             │ │

│  │    Live cards │ Temperature charts │ History table      │ │

│  └───────────────────────┬─────────────────────────────────┘ │

│                          │ HTTP REST (JSON)                  │

│  ┌───────────────────────▼─────────────────────────────────┐ │

│  │           FLASK API BACKEND  (Python 3.x)               │ │

│  │  LSTM inference · Hysteresis control · State manager    │ │

│  │  /predict  /status  /control  /history  /health         │ │

│  └────────┬──────────────────────────┬──────────────────── ┘ │

└───────────┼──────────────────────────┼──────────────────────┘

│ TRANSPORT LAYER          │

│ HTTP GET /temperature    │ HTTP GET /fan?state=on|off

┌───────────▼──────────┐   ┌──────────▼──────────────────────┐

│  PERCEPTION LAYER    │   │  PERCEPTION LAYER               │

│  ESP32 + DHT22       │   │  ESP32 + Relay Module           │

│  (Sensor Node)       │   │  (Actuator Node)                │

│  IP: 10.39.146.79    │   │  IP: 10.39.146.232              │

└──────────────────────┘   └─────────────────────────────────┘

Fig. 1. Complete three-tier system architecture.

A. Hardware Components

Table I — Hardware Component Summary

| Component | Function | Key Specification |
| --- | --- | --- |
| ESP32 Dev Board (×2) | Microcontroller | 240 MHz dual-core, Wi-Fi 802.11 b/g/n, 4 MB Flash |
| DHT22 Sensor | Temp. & Humidity acquisition | ±0.5°C temp., ±2–5% RH accuracy |
| 4-Channel Relay Module | Fan switching | 5 V control signal, 10 A / 250 VAC load |
| 12 V DC Cooling Fan | Air circulation / cooling | Variable speed, switched via relay |

B. Sensor Node (ESP32 + DHT22)

The sensor ESP32 runs an Arduino-framework HTTP server that exposes a /temperature endpoint. On each HTTP GET request, the DHT22 is polled and a JSON response is returned:

{ "temperature": 18.4, "humidity": 62.1 }

The DHT22 was selected over the lower-cost DHT11 for its significantly superior accuracy (±0.5°C versus ±2°C), which is critical given the ±0.5°C hysteresis band used in the control algorithm.

C. Fan Controller Node (ESP32 + Relay)

The second ESP32 exposes a /fan?state={on|off} endpoint. GPIO pins 5, 18, 26, and 27 are all configured as relay outputs and driven simultaneously, providing redundant actuation paths. Active-high relay logic is used (RELAY_ON = HIGH). The node also exposes a /status endpoint returning JSON with uptime, fan state, and active-pin information.

D. Flask REST API Backend

The Python backend (api.py) is the central intelligence and coordination hub. Five functions are performed concurrently:

LSTM Inference — Generates the optimal temperature prediction for the current time slot.

Sensor Polling — Fetches live temperature and humidity over HTTP from the sensor ESP32.

Hysteresis Control — Compares measured versus predicted temperature and commands the fan relay.

State Management — Maintains a thread-safe rolling 24-entry history buffer.

REST API — Exposes structured JSON endpoints for dashboard consumption.

A background daemon threading.Thread executes the control cycle every 2 seconds. Shared runtime state is protected by a threading.Lock to eliminate race conditions between the control thread and HTTP request handlers.

E. Web Dashboard

An HTML/JavaScript single-page application polls the Flask API periodically and renders:

• Live temperature, humidity, and fan-state indicator cards.

• Predicted vs. actual temperature comparison.

• Line chart of the last 24 control-cycle readings.

• Manual fan override controls.

IV. Machine Learning Model

A. Dataset Description

The model was trained on final_saffron_temperature_dataset.csv, containing multi-year hourly temperature recordings pertaining to the saffron-growing microclimate. The dataset underwent feature engineering to produce 10 input features (Table II).

Table II — LSTM Input Feature Set

| Feature | Type | Description |
| --- | --- | --- |
| `temperature` | Continuous | Ambient temperature (°C) — prediction target |
| `hour_sin` | Cyclic | sin(2π · hour / 24) |
| `hour_cos` | Cyclic | cos(2π · hour / 24) |
| `month_sin` | Cyclic | sin(2π · month / 12) |
| `month_cos` | Cyclic | cos(2π · month / 12) |
| `temp_lag_1` | Lag | Temperature 1 hour prior |
| `temp_lag_3` | Lag | Temperature 3 hours prior |
| `temp_lag_6` | Lag | Temperature 6 hours prior |
| `temp_lag_12` | Lag | Temperature 12 hours prior |
| `temp_lag_24` | Lag | Temperature 24 hours prior |

Cyclic encoding of hour and month preserves the topological continuity of periodic features (e.g., hour 23 is adjacent to hour 0), preventing the artificial boundary artifacts that arise with raw integer encoding [11]. All 10 features were normalized to [0, 1] using Min-Max scaling prior to model input.

B. LSTM Architecture

The network architecture comprises two stacked LSTM layers with Dropout regularization, followed by a single Dense output node (Table III).

Table III — LSTM Model Architecture

| Layer | Units | Output Shape | Parameters |
| --- | --- | --- | --- |
| LSTM (return_sequences=True) | 128 | (batch, 24, 128) | 71,168 |
| Dropout | 0.2 | (batch, 24, 128) | 0 |
| LSTM | 64 | (batch, 64) | 49,408 |
| Dropout | 0.2 | (batch, 64) | 0 |
| Dense | 1 | (batch, 1) | 65 |
| **Total** |  |  | **120,641** |

A lookback window of 24 time steps (24 hours) captures one complete diurnal cycle, providing the model with sufficient temporal context to learn both intra-day and night-day transition patterns.

C. Training Configuration

| Hyperparameter | Value |
| --- | --- |
| Optimizer | Adam (default learning rate = 0.001) |
| Loss Function | Mean Squared Error (MSE) |
| Maximum Epochs | 80 |
| Batch Size | 32 |
| Validation Split | 10% of training set |
| Early Stopping Patience | 10 epochs (restore best weights) |
| Train / Test Split | 80% / 20% |

Early stopping with weight restoration prevents overfitting while reducing unnecessary computation once validation loss plateaus.

D. Inference Pipeline

At runtime, single-step inference is performed as follows:

Extract current hour and month from the system clock.

Compute cyclic features: hour_sin, hour_cos, month_sin, month_cos.

Populate lag features with an internal reference temperature (10.0°C seed value).

Scale the 10-element input vector using the pre-fitted MinMaxScaler.

Reshape to (1, 1, 10) for LSTM input.

Run forward inference; apply inverse Min-Max transform to recover °C prediction.

This design allows stateless API prediction calls without maintaining a live rolling buffer of sensor history, at the cost of lag feature approximation during online operation.

V. Control Logic

A. Hysteresis-Based On/Off Control

To prevent excessive relay cycling near the setpoint, a symmetric hysteresis dead band of ±0.5°C is implemented. The decision logic is:

IF current_temp > predicted_temp + 0.5°C  →  Fan ON

IF current_temp < predicted_temp − 0.5°C  →  Fan OFF

ELSE                                        →  Hold (no state change)

This ensures the relay is toggled only when the temperature deviates meaningfully from the LSTM-predicted optimal setpoint, reducing mechanical wear and oscillation artefacts.

B. Threaded Control Architecture

The control loop runs as a daemon thread with a 2-second polling interval, fully decoupled from Flask's HTTP request handling. The runtime state dictionary (temperature, humidity, prediction, fan state, history) is protected by a threading.Lock, guaranteeing thread-safe reads for dashboard API calls concurrent with write operations from the control thread.

C. Fault Tolerance

If the sensor ESP32 is unreachable (e.g., network disruption or device power loss):

• The control cycle is aborted for that iteration.

• The sensor_online flag is set to False, surfaced in the /status API.

• The fan state is not modified, defaulting to its last known state.

• The error is logged to stdout for operator diagnosis.

This fail-safe design prevents erroneous actuation in the absence of valid sensor data.

VI. Experimental Results

A. Model Performance

The trained LSTM model was evaluated on the held-out 20% test partition, comprising 2,896 samples. Predictions were inverse-transformed to °C before metric computation.

Table IV — LSTM Model Performance on Test Set (n = 2,896)

| Metric | Value |
| --- | --- |
| Mean Absolute Error (MAE) | **0.3523 °C** |
| Root Mean Squared Error (RMSE) | **0.4960 °C** |
| Coefficient of Determination (R²) | **0.9920** |

These results demonstrate strong predictive fidelity. An MAE of 0.3523°C is well below the ±0.5°C hysteresis threshold, ensuring that the control algorithm operates based on reliable predictions. The R² of 0.9920 indicates that the model explains 99.2% of variance in the test temperature series.

B. Control System Latency

Table V — End-to-End Latency Breakdown

| Operation | Measured Latency |
| --- | --- |
| Sensor HTTP read (LAN) | < 200 ms |
| LSTM inference (CPU) | < 50 ms |
| Fan command HTTP (LAN) | < 200 ms |
| Full control cycle | < 500 ms |

The 2-second control interval significantly exceeds the worst-case cycle latency, guaranteeing that each iteration completes before the next is triggered.

C. System Resource Utilisation

The Flask backend runs comfortably on a standard laptop or Raspberry Pi class device. The LSTM model file (saffron_temperature_lstm_model.h5) is 1.4 MB, and single-step inference consumes negligible CPU time (< 50 ms), making edge deployment on low-power hardware feasible.

VII. Discussion

A. Predictive vs. Reactive Control

The LSTM-based predictive approach offers a fundamental advantage over threshold thermostats: the fan state is governed by an anticipated optimal temperature rather than crossing a fixed boundary. This enables the system to pre-condition the environment before the target temperature is exceeded, reducing peak thermal excursions.

B. System Strengths

• High prediction accuracy (R² = 0.9920) validated on real data.

• Low cost — complete hardware stack assembled for under $20 USD.

• Wireless architecture — no field wiring required for sensor-to-actuator communication.

• Offline-capable — all inference runs locally; no cloud dependency.

• Extensible — additional ESP32 sensor-actuator pairs can be added to the network without redesigning the backend.

C. Limitations

Lag feature approximation: The online inference pipeline approximates lag features with a fixed seed value (10°C) rather than a true 24-hour rolling buffer of live readings. Under sustained rapid temperature changes, this approximation may introduce systematic prediction error.

Static model: The LSTM is not updated post-deployment. Distributional drift (e.g., climate seasonality in a new year or a different growing site) would require periodic retraining.

Cooling only: The single-actuator design addresses only overheating. Cold-night scenarios (below the 5°C threshold during flowering) are not addressed.

Network dependency: All inter-component communication relies on a shared Wi-Fi LAN. Network outages disable sensor reads and fan commands simultaneously.

Security: Plain HTTP without authentication is unsuitable for internet-exposed deployments.

D. Comparison with Existing Approaches

Table VI — Comparison with Related Systems

| Criterion | Proposed System | Threshold Thermostat | Cloud IoT + ML |
| --- | --- | --- | --- |
| Predictive control | ✅ LSTM (R²=0.992) | ❌ Reactive | ✅ (cloud-side) |
| Local/edge inference | ✅ | ✅ | ❌ |
| Hardware cost | ~$20 USD | ~$5–15 USD | High (subscription) |
| Network dependency | LAN only | None | Internet required |
| Dashboard / monitoring | ✅ Web UI | ❌ | ✅ |
| Offline operation | ✅ | ✅ | ❌ |
| Bidirectional temperature control | ❌ | Configurable | ✅ |

VIII. Future Work

The following enhancements are identified for future development:

Live 24-hour rolling buffer — Maintain a circular buffer of real sensor readings to provide accurate lag features during online inference, replacing the seed-value approximation.

Bidirectional climate control — Add a heater relay to address cold-night scenarios, enabling full closed-loop regulation across the complete saffron temperature envelope.

Online / periodic retraining — Implement an automated retraining pipeline that incorporates accumulated sensor data to counteract model drift.

MQTT-based communication — Replace REST polling with a publish-subscribe MQTT broker (e.g., Mosquitto) for lower latency and reduced network overhead.

TensorFlow Lite edge inference — Quantise and deploy the LSTM model directly on an ESP32 using TensorFlow Lite for Microcontrollers, eliminating the server dependency.

Multi-zone control — Extend the architecture to support independent sensor-actuator pairs for zone-specific greenhouse management.

Security hardening — Implement HTTPS with TLS, API-key authentication, and mutual certificate verification for production deployments.

Explainable AI integration — Apply SHAP (SHapley Additive exPlanations) to produce interpretable feature importance scores for LSTM predictions, aiding agronomist trust and adoption.

IX. Conclusion

This paper has presented and reviewed a complete AI-integrated IoT system for saffron temperature control. The system combines distributed ESP32-based sensing and actuation, LSTM deep learning trained with temporal feature engineering, and a Flask-based RESTful control backend into a cohesive, low-cost, locally-deployed precision-agriculture solution. Evaluation on a held-out test set of 2,896 samples demonstrates strong predictive performance: MAE of 0.3523°C, RMSE of 0.4960°C, and R² of 0.9920. The hysteresis-based control algorithm ensures stable and efficient actuator operation, while the multi-threaded backend provides concurrent, responsive handling of control and monitoring tasks.

The system demonstrates the practical viability of integrating accessible embedded hardware with open-source deep learning to produce intelligent, field-deployable smart greenhouse controllers. With the enhancements described in Section VIII—particularly live lag buffering, bidirectional climate control, and MQTT integration—the architecture can be evolved from a research prototype into a production-grade system suitable for commercial saffron cultivation.

References

[1] S. Wolfert, L. Ge, C. Verdouw, and M. J. Bogaardt, "Big data in smart farming – A review," Agricultural Systems, vol. 153, pp. 69–80, 2017.

[2] F. Gresta, G. M. Lombardo, L. Siracusa, and G. Ruberto, "Saffron, an alternative crop for sustainable agricultural systems. A review," Agronomy for Sustainable Development, vol. 28, no. 1, pp. 95–112, 2008.

[3] N. Gondchawar and R. S. Kawitkar, "IoT based smart agriculture," International Journal of Advanced Research in Computer and Communication Engineering, vol. 5, no. 6, pp. 838–842, 2016.

[4] V. C. Patil and A. D. Kadu, "IOT based smart agriculture monitoring and controlling system," International Research Journal of Engineering and Technology, vol. 3, no. 5, 2016.

[5] A. Tzounis, N. Katsoulas, T. Bartzanas, and C. Kittas, "Internet of Things in agriculture, recent advances and future challenges," Biosystems Engineering, vol. 164, pp. 31–48, 2017.

[6] Espressif Systems, ESP32 Series Datasheet v3.5, Espressif Systems, 2023.

[7] S. Hochreiter and J. Schmidhuber, "Long short-term memory," Neural Computation, vol. 9, no. 8, pp. 1735–1780, 1997.

[8] D. Liu, A. K. Mishra, Z. Yu, H. Yang, and S. Lv, "Soil moisture uncertainty in hydrologic modeling with a quasi-linear effective rainfall model," Journal of Hydrology, vol. 559, pp. 386–394, 2018.

[9] M. Azaza, C. Tanougast, E. Fabrizio, and A. Mami, "Smart greenhouse fuzzy logic based control system enhanced with wireless data monitoring," ISA Transactions, vol. 61, pp. 297–307, 2016.

[10] H. Navarro-Hellín, J. M. del Rincon, R. Domingo-Miguel, F. Soto-Valles, and R. Torres-Sánchez, "A decision support system for managing irrigation in agriculture," Computers and Electronics in Agriculture, vol. 124, pp. 121–131, 2016.

[11] D. Peng, Y. Wu, and H. Daume, "Time-series feature encoding for neural networks," arXiv preprint arXiv:1905.09659, 2019.

[12] R. V. Molina, M. Valero, Y. Navarro, J. L. Guardiola, and A. García-Luis, "Temperature effects on sprouting and flowering in Crocus sativus L.," Scientia Horticulturae, vol. 103, no. 3, pp. 361–379, 2005.